Valsts SIA „Latvijas Televīzija” konkursa

„Par dokumentālu īsfilmu „LATVIJAI 95. LATVIJA ŠODIEN” izveidi”

nolikuma

1.pielikums

Filmu tehniskie parametri

|  | MPEG2 | MXF D-10 | QuickTime |
| --- | --- | --- | --- |
| File extension | mpg | mxf (OP1a) | mov |
| Video |  |  |  |
| Format | MPEG2 | MPEG-2 IMX | MPEG-2 IMX |
| Stream type | Program stream | Program stream | Program stream |
| Profile@Level | Main@Main or High@Main | 4:2:2@Main | 4:2:2@Main |
| GOP length | 12 | I frame | I frame |
| Bit rate mode | Constant | Constant | Constant |
| Bit rate | 15 Mbps | 50Mbps | 50 Mbps |
| Display aspect ratio | 16:9 | 16:9 | 16:9 |
| Frame rate | 25 fps | 25 fps | 25 fps |
| Video standard | PAL (720x576) | PAL (720x576) | PAL (720x576) |
| Color space | YUV | YUV | YUV |
| Chroma subsampling | 4:2:2 or 4:2:0 | 4:2:2 | 4:2:2 |
| Bit depth | 8 bits | 8 bits | 8 bits |
| Scan type | Interlaced | Interlaced | Interlaced |
| Scan order | Top Field First | Top Field First | Top Field First |
| Audio |  |  |  |
| Track count | 1 | 1 | 1 |
| Format | MPEG-1 Layer 2 | PCM | PCM |
| Bit rate mode | Constant | Constant | Constant |
| Bit rate | 384 Kbps | 1 536 Kbps | 1 536 Kbps |
| Channels | 2 channels (stereo) | 2 channels (stereo) | 2 channels (stereo) |
| Sampling rate | 48.0 kHz | 48.0 kHz | 48.0 kHz |
| Bit depth | 16 bits | 16 or 24 bits | 16 or 24 bits |
| Alignment level | -18 dBFS | -18 dBFS | -18 dBFS |
| Peak level | -9 dBFS | -9 dBFS | -9 dBFS |
| Bit depth | 16 bits | 16 or 24 bits | 16 or 24 bits |
| Alignment level | -18 dBFS | -18 dBFS | -18 dBFS |
| Peak level | -9 dBFS | -9 dBFS | -9 dBFS |